A Simulation Study

on

Adaptive Traffic Light Control

Dipun Wickramaratne

15.12.2024

CSC 446 Simulation Study

Dr. Kui Wu

Contents

1 Project Description	1

2 Simulation Model	1

3 Simulation Parameters	2

4 Methodology	3

4.1 Simulation Parameter Setup	4

4.2 Statistical Analysis	4

5 Analysis	5

5.1 Data	5

5.2 Mean, Standard Deviation and Confidence Intervals	7

5.3 Scenario Analysis	9

6 Conclusion	10

6.1 Results	10

6.2 Improvements to The System	10

6.3 Summary	10

7 Source Code	11

# 1 Project Description

This simulation study focuses on the traffic flow at a signal-controlled intersection to analyse the efficiency of two different types of strategies such as fixed-time and adaptive-time systems. The two systems act differently where the fixed-time operation handles traffic flow with a predetermined green light duration, while the adaptive-time operation handles traffic flow by dynamically adjusting the green light duration based on the traffic volume and the queue lengths. The simulation is conducted under various traffic scenarios to emulate the best possible scenarios in the real world, including rush hour traffic, night-time low traffic, and normal traffic conditions. The primary goal of this study is to determine which system offers better performance under different traffic conditions and provide reasons for the potential benefits of implementing an adaptive traffic control system.

# 2 Simulation Model

The simulation model is setup to be a signal-controlled intersection with two traffic flows, north-west and east-west, each managed by a traffic system. Vehicles arrive at each direction on a random distribution depending on the traffic flow scenario (i.e. rush hour traffic, night-time low traffic and normal traffic). However, when maintaining queues for each system there are a few rules that defer from each other. For instance, a fixed-time system has a separate queue for a left turn and a separate green light time that happens at the start of every direction which is followed by the straight-line queue and its respective green light time after the left turn green light time is done. Retrospectively, an adaptive-time system has no separate time frame for a left turn and instead has one queue which consists of both left turn and straight queue. In this case the left turn is done when the driver is safe to do so. In both systems a right turn is part of the straight queue, and it is done with the discretion of the driver and when it is safe to do so.

During each simulation cycle, vehicles are generated and added to their respective queues and then the system serves the queues according to their own strategy in which they update queue lengths, total waiting time and the number of cars served. Then the simulation tracks performance metrics such as total cars served, average waiting time, average queue length and traffic flow efficiency to evaluate the effectiveness of the system.

# 3 Simulation Parameters

The table below provides a description of the variables used in the simulation.

| Variable | Description |
| --- | --- |
| self.direction | This parameter defines the direction of the flow of traffic |
| self.destination_lane | This parameter indicates the target lane for the vehicles |
| self.queue_straight | This parameter defines the number of people in the straight queue |
| self.queue_left | This parameter defines the number of people in the queue for a left turn |
| self.total_waiting_time | This parameter calculates the total waiting time in the queue |
| self.total_cars_served | This parameter calculates the total cars served in the system |
| self.total_cars_generated | This parameter calculates the total cars generated that needs to be served |
| self.cycle_queue_lengths | This parameter calculates the queue length for each cycle |
| cars_served_rate | This parameter represents the number of vehicles that can pass through a given time |
| green_time | Has two parameters fixed and fixed left which consist of time allocated for straight moving traffic and left turn traffic |
| fixed_cars_served | This parameter finds the number of cars that can be served in each green time length for a fixed-time system |
| adaptive_cars_served | This parameter finds the number of cars that can be served in each green time length for a adaptive-time system |

# 4 Methodology

The simulation models a simplified traffic light system for an intersection with two directions, north-south and east-west. It is designed to perform two different systems, fixed-time and adaptive-time, to handle varying traffic flow conditions. Here's the breakdown of how the simulation works:

The simulation begins with the initialization of the TrafficLight class. First, we introduce the main attributes described in the simulation parameters. Then, we create a function called generate_traffic to generate the traffic. This function takes inputs such as seed, mode, and traffic_type. It uses different traffic_type values to determine the arrival ranges for cars going straight and turning left based on the chosen mode (fixed or adaptive). The function updates the TrafficLight instance by modifying the queue_straight, queue_left, and total_cars_generated attributes.

Next, the TrafficLight class includes another function called serve_traffic, which takes inputs like mode, direction, and traffic_type to manage the generated traffic. In fixed mode, the traffic light uses predefined values for green_straight_time and green_left_time from the green_time dictionary. The number of cars served is determined by a predefined service rate (fixed_cars_served), representing how many vehicles can clear the intersection during the fixed green light durations. The method to calculate the number of left-turn vehicles served is based on the queue_left and the allowable service limit (served['left']). A similar process is applied to the straight-moving queue. The total_cars_served attribute is incremented to reflect the number of cars served for both left turn (only for fixed-time system) and straight queues

In adaptive mode, the green light time dynamically adjusts based on queue size. If traffic is heavy, the duration of the green light increases, and if traffic is light or absent, the duration decreases. This adjustment applies to each traffic_type.

The total_waiting_time is updated to account for the time spent in queues during each cycle. In fixed mode, both left-turn and straight green durations are considered, whereas in adaptive mode, only the straight green durations are considered. At the end of each cycle, the total queues for both straight and left-turn traffic are stored in cycle_queue_lengths.

To orchestrate the entire process of simulating traffic, we introduce a new function called simulate_traffic. This function takes inputs such as cycles, mode, north_south, east_west, seed, and traffic_type. For each cycle, new cars are added to the queues for both directions using the generate_traffic method. To ensure variability, north_south_seed and east_west_seed are assigned different seed values. Once new cars are added to the queues, the function calls serve_traffic for the given mode and traffic_type.

Following this, performance metrics such as total_cars_served, average_waiting_time, average_queue_length, and traffic_flow_efficiency are calculated. These results are returned as a dictionary called result. (Traffic flow efficiency is measured as the percentage of generated cars that were successfully served during the simulation.)

Lastly, we introduce a new function called run_multiple_simulations to repeat the simulation multiple times. This function takes inputs such as runs, cycles, mode, and traffic_type. It starts by creating an empty list called results. Using a for loop, the simulation is run for a specified number of times, with each run using a unique random seed to ensure variability. For each run, both directions (north-south and east-west) are initialized. The results of each simulation, obtained from the simulate_traffic function, are stored in the results list.

## 4.1 Simulation Parameter Setup

The main parameters used in this simulation in the cars_served_rate, green_time, fixed_cars_served, and adaptive_cars_served. We assumed that the number of cars served per 10 seconds to be 0.3 and we introduced the green light time for straight traffic to have 30 seconds and 10 seconds for a left turn. Using these values, we found the service rate for fixed and adaptive modes.

## 4.2 Statistical Analysis

In our simulation we gathered data on four different metrics which was total_cars_served, average_waiting_time, average_queue_length and traffic_flow_efficiency which was calculated in the function simulate_traffic

# 5 Analysis

## 5.1 Data

The tables below provide the data collected for 4 different metrics in 5 different runs for six different scenarios. The scenarios consist of a normal traffic flow for both fixed and adaptive, a rush hour traffic flow for both fixed and adaptive, and a low traffic flow for both fixed and adaptive.

Table 1: Fixed-Time Normal Traffic Results

| Run # | Total Cars Served | Average Waiting Time | Average Queue Length | Traffic Flow Efficiency % |
| --- | --- | --- | --- | --- |
| 1 | 240 | 40 | 45 | 66.7 |
| 2 | 190 | 40 | 28 | 73.1 |
| 3 | 240 | 40 | 45 | 66.7 |
| 4 | 190 | 40 | 12 | 95.0 |
| 5 | 210 | 40 | 18 | 87.5 |

Table 2: Adaptive-Time Normal Traffic Results

| Run # | Total Cars Served | Average Waiting Time | Average Queue Length | Traffic Flow Efficiency % |
| --- | --- | --- | --- | --- |
| 1 | 240 | 47 | 12 | 100.0 |
| 2 | 207 | 36 | 12 | 98.8 |
| 3 | 230 | 43 | 12 | 100.0 |
| 4 | 138 | 24 | 11 | 92.0 |
| 5 | 218 | 37 | 18 | 90.8 |

Table 3: Fixed-Time Rush Hour Results

| Run # | Total Cars Served | Average Waiting Time | Average Queue Length | Traffic Flow Efficiency % |
| --- | --- | --- | --- | --- |
| 1 | 240 | 80 | 128 | 36.3 |
| 2 | 240 | 80 | 109 | 40.7 |
| 3 | 240 | 80 | 128 | 36.4 |
| 4 | 240 | 80 | 84 | 48.0 |
| 5 | 240 | 80 | 119 | 38.1 |

Table 4: Adaptive-Time Rush Hour Results

| Run # | Total Cars Served | Average Waiting Time | Average Queue Length | Traffic Flow Efficiency % |
| --- | --- | --- | --- | --- |
| 1 | 402 | 79 | 42 | 87.8 |
| 2 | 444 | 74 | 93 | 63.4 |
| 3 | 441 | 73 | 47 | 82.1 |
| 4 | 396 | 72 | 26 | 96.0 |
| 5 | 399 | 69 | 55 | 76.7 |

Table 5: Fixed-Time Low Traffic Results

| Run # | Total Cars Served | Average Waiting Time | Average Queue Length | Traffic Flow Efficiency % |
| --- | --- | --- | --- | --- |
| 1 | 120 | 40 | 6 | 100.0 |
| 2 | 120 | 40 | 6 | 100.0 |
| 3 | 70 | 40 | 4 | 100.0 |
| 4 | 50 | 40 | 3 | 100.0 |
| 5 | 100 | 40 | 5 | 100.0 |

Table 6: Adaptive-Time Low Traffic Results

| Run # | Total Cars Served | Average Waiting Time | Average Queue Length | Traffic Flow Efficiency % |
| --- | --- | --- | --- | --- |
| 1 | 108 | 19 | 8 | 90.0 |
| 2 | 56 | 12 | 4 | 93.8 |
| 3 | 99 | 17 | 8 | 90.2 |
| 4 | 68 | 12 | 4 | 96.6 |
| 5 | 70 | 13 | 4 | 100.0 |

## 5.2 Mean, Standard Deviation and Confidence Intervals

The table below shows the mean, standard deviation and confidence intervals for the six scenarios mentioned in the data section.

The sample mean is calculated using the following formula where n is the number of runs and xi is the value of the chosen metric value for a given run i.

The standard deviation is calculated using the following formula where N is the number of runs and xi is the value of the chosen metric value for a given run i.

The standard error is calculated using the following equation where s is the standard deviation and n is the number of runs

Then the confidence interval (CI) is calculated using the following formula where SE is the standard error and alpha value of 0.05 and the t value is calculated using a t-distribution table

Table 7: Fixed-Time Normal Traffic Results

| Type | Total Cars Served | Average Waiting Time | Average Queue Length | Traffic Flow Efficiency % |
| --- | --- | --- | --- | --- |
| Mean | 214 | 40 | 29.95 | 77.8 |
| S.D. | 25.1 | 40 | 14.94 | 12.85 |
| Standard Error | 11.22 | 0 | 6.68 | 5.85 |
| Confidence Interval (CI) | (182.83, 245.17) | (40, 40) | (11.3, 48.5) | (61.8, 93.7) |

Table 8: Adaptive-Time Normal Traffic Results

| Type | Total Cars Served | Average Waiting Time | Average Queue Length | Traffic Flow Efficiency % |
| --- | --- | --- | --- | --- |
| Mean | 206.6 | 37.27 | 13.22 | 96.32 |
| S.D. | 40.31 | 8.62 | 2.97 | 4.52 |
| Standard Error | 18.03 | 3.86 | 1.33 | 2.02 |
| Confidence Interval (CI) | (156.55, 256.65) | (26.56,47.97) | (9.53,16.91) | (90.7,101.93) |

Table 9: Fixed-Time Rush Hour Results

| Type | Total Cars Served | Average Waiting Time | Average Queue Length | Traffic Flow Efficiency % |
| --- | --- | --- | --- | --- |
| Mean | 240 | 80 | 113.2 | 39.9 |
| S.D. | 0 | 0 | 18.39 | 4.86 |
| Standard Error | 0 | 0 | 8.22 | 2.17 |
| Confidence Interval (CI) | (240,240) | (80,80) | (90.37,136.02) | (33.86,45.93) |

Table 10: Adaptive-Time Rush Hour Results

| Type | Total Cars Served | Average Waiting Time | Average Queue Length | Traffic Flow Efficiency % |
| --- | --- | --- | --- | --- |
| Mean | 439.8 | 73.3 | 52.85 | 81.19 |
| S.D. | 22 | 3.67 | 24.62 | 12.24 |
| Standard Error | 9.84 | 1.64 | 11.01 | 5.47 |
| Confidence Interval (CI) | (412.47, 467.12) | (68.75,77.85) | (22.8,83.4) | (65.98, 96.39) |

Table 11: Fixed-Time Low Traffic Results

| Type | Total Cars Served | Average Waiting Time | Average Queue Length | Traffic Flow Efficiency % |
| --- | --- | --- | --- | --- |
| Mean | 92 | 40 | 4.6 | 100 |
| S.D. | 31.14 | 0 | 1.56 | 0 |
| Standard Error | 13.93 | 0 | 0.7 | 0 |
| Confidence Interval (CI) | (53.32, 130.67) | (40,40) | (2.67, 6.53) | (100,100) |

Table 12: Adaptive-Time Low Traffic Results

| Type | Total Cars Served | Average Waiting Time | Average Queue Length | Traffic Flow Efficiency % |
| --- | --- | --- | --- | --- |
| Mean | 80.2 | 14.53 | 5.84 | 94.12 |
| S.D. | 22.16 | 3.17 | 2.31 | 4.28 |
| Standard Error | 9.91 | 1.42 | 1.03 | 1.91 |
| Confidence Interval (CI) | (52.68, 107.72) | (10.59, 18.47) | (2.97, 8.71) | (88.81, 99.42) |

## 5.3 Scenario Analysis

When comparing the two systems in terms of traffic flow, we can identify which system is better suited for different traffic conditions.

First, consider the fixed-time system and the adaptive-time system under normal traffic flow (Table 7, Table 8). The fixed-time system shows a higher rate of total cars served, whereas the adaptive-time system performs better in terms of average waiting time, average queue length, and traffic flow efficiency.

Next, consider the fixed-time system and the adaptive-time system during rush hour traffic flow (Table 9, Table 10). The adaptive-time system outperforms the fixed-time system across all metrics, including total cars served, average waiting time, average queue length, and traffic flow efficiency.

Finally, examine the fixed-time system and the adaptive-time system under low traffic flow conditions (Table 11, Table 12). In this scenario, the fixed-time system achieves better results for total cars served, average queue length, and traffic flow efficiency. However, it has a longer average waiting time compared to the adaptive-time system.

# 6 Conclusion

## 6.1 Results

The results of our simulation indicate that the adaptive-time system is more beneficial for a normal traffic flow and rush hour traffic flow with having a significant difference in traffic flow efficiency compared to a fixed-time system. This suggests that considering one queue for straight, left, and right turns would be a good implementation for a traffic system in those conditions. However, under low traffic flow conditions, the fixed-time system achieved better results for total cars served, queue length and traffic flow efficiency despite of having longer waiting time. However, the higher waiting times associated with the fixed-time system in low traffic scenarios make the adaptive-time system a viable alternative, given the negligible differences in other metrics.

## 6.2 Improvements to The System

A notable observation the system we propose is that the traffic flow efficiency does not consistently reach a very high rate. Improvements can be made by fine-tuning the green light durations based on specific queue lengths, which could enhance performance under varying traffic conditions, particularly during rush hours. Furthermore, we think the addition of more options in the adaptive-time system for the low traffic flow conditions may make the system more universally applicable in such conditions. Future enhancements could also consider incorporating real-world complexities, such as pedestrian crossings, vehicle accidents that would cause a massive hold up, and emergency vehicle priority where it stops the traffic from all directions.

## 6.3 Summary

In summary, this simulation study illustrates the performance of fixed-time and adaptive-time systems under diverse traffic conditions. The observations suggest that the adaptive-time system is more beneficial in high and normal traffic flows. However, its performance in low traffic conditions need refinements to make the system more efficient. The simulation study also acknowledges that its primary focus is on vehicles, but in future studies the consideration of the real world complexities in a given traffic flow system would result in a more realistic and a practical model.

# 7 Source Code

Source code will be provided with the submission